# Penalty-Based Investment Allocation Models for CPAT: A Transparent Alternative to Logit-Based Share Rules

2025-05-11

Table of Contents

## Highlights

- Introduces velocity- and acceleration-penalised allocation models as alternatives to logit rules.

- Provides analytical steady-state shares and transition paths suitable for CPAT.

- Embeds system costs (flexibility, integration, physical constraints) in a unified framework.

- Produces spreadsheet-ready formulae for total effective LCOE and update rules.

- Offers calibration guidance and policy-relevant interpretations for fiscal and LTS work.

# Introduction

The allocation of investment across competing power-generation technologies is one of the most consequential decisions in long-term energy planning. It determines not only the cost, reliability, and environmental footprint of the electricity system, but also the macroeconomic and fiscal conditions under which countries pursue development and climate goals. Over the past several decades, a wide range of modelling paradigms has emerged to support these decisions, each reflecting a distinct view of how technology choices are made and how transitions unfold. Some are grounded in economic optimisation, others in engineering accounting, others in behavioural choice theory, and still others in evolutionary competition among technologies. Understanding their respective strengths and weaknesses is a prerequisite for designing allocation rules suitable for policy-focused tools such as the Climate Policy Assessment Tool (CPAT).

The most widely used approach in energy-system planning is least-cost optimisation. Frameworks such as TIMES, MARKAL, MESSAGE or OSeMOSYS represent the energy system as a set of interlinked technologies, resources and constraints, and compute investment and dispatch paths that minimise the discounted cost of supplying energy. In simplified form, these models choose capacity for each technology and hourly generation levels so as to minimise total fixed and variable costs, subject to three core constraints: (i) hourly generation must meet hourly demand; (ii) generation from each technology cannot exceed its installed capacity adjusted by its availability in each hour; and (iii) total firm capacity must satisfy a planning reserve margin to ensure reliability.

The dual variables associated with the energy-balance and reliability constraints naturally define system-level marginal costs. These system costs—reflecting curtailment, flexibility limits, or reliability burdens—can be incorporated into reduced-form planning models as an additive “system cost” term appended to each technology’s private levelised cost [@grubb2019; @miller2023]. This reduced-form mapping motivates the treatment of effective costs in models such as CPAT.

A second major paradigm is represented by accounting frameworks such as LEAP [@leap2020] and Sisepuede [@sisepuede2024], which emphasise transparent, user-defined scenario construction rather than endogenous optimisation or behavioural response. These systems excel at incorporating sectoral detail, stakeholder guidance and narrative consistency. However, because technology shares are exogenously specified, they cannot simulate the response of investment behaviour to changing policy or cost signals. For a tool such as CPAT, which must trace how carbon prices, taxes or subsidies propagate through investment patterns, this absence of behavioural structure remains a central limitation [@CPATdocs].

A contrasting behavioural approach is found in the evolutionary dynamics of the FTT:Power framework [@Mercure2012]. Here, technologies compete for market share according to a pairwise substitution mechanism shaped by relative costs, learning effects and path dependence. In stylised form, the governing equation is:

dsidt=j≠i​si sj Fij(Cj−Ci),

where si and sj are the shares of total capacity held by technologies i and j, dsidt is the corresponding change in the capacity share of technology i, and Ci and Cj denote their respective generalised costs. The pairwise choice probability Fij is represented by a logit preference function: {Fij(Cj−Ci)=11+exp​Ci−Cjσ} where σ captures the dispersion of investor preferences. This formulation produces S-curve diffusion, lock-in, and path dependence.

Against this landscape, CPAT adopts a discrete-choice formulation based on the multinomial logit model [@CPATdocs], in which investment shares respond smoothly to differences in levelised cost:

si=exp(−Ci/β)j​exp(−Cj/β).

{#eq-CPATlogit} This mapping is computationally trivial, transparent to calibrate and captures the intended directional response of investment to policy shocks. In operational use within CPAT, Ci is not simply the private levelised cost: it incorporates (i) system integration costs associated with variable renewables (derived from storage and balancing needs), (ii) a calibration term capturing historical coal–gas relative competitiveness. In addition a hard scaling constraint on annual renewable capacity additions (which reflects empirical construction bottlenecks) is imposed. These adjustments enable CPAT to mimic, in reduced form, several features that would arise endogenously in a more detailed hourly optimisation.

Recent work in climate economics introduces yet another perspective. @BauerHallegatteMcIsaac2025 show that institutional or political frictions can be represented via quadratic penalties that modify marginal abatement allocation. In a single-period version of their framework, an optimal allocation of abatement ai across sectors solves:

min{ai}i​Ciai+λ2ai2 s.t. i​ai=A,

{#eq-hallegatte} yielding closed-form expressions that balance cost and institutional feasibility. This motivation underpins the velocity- and acceleration-penalised allocation rules proposed in this paper.

Taken together, these modelling traditions motivate the search for an allocation mechanism that preserves the interpretability and computational lightness of the logit rule, while allowing corner solutions, representing diversification pressures and encoding simple transition frictions. This paper develops precisely such mechanisms. The velocity-penalised model produces a closed-form investment allocation in which high-cost technologies may receive zero share, while the acceleration-penalised model limits abrupt year-to-year reallocations. Both models can be integrated directly into CPAT, complementing its existing system-cost adjustments and renewable scale-up constraints.

The remainder of the paper introduces a standard least-cost optimisation model before velocity or acceleration effects. It then situates the two adjustment-cost alternatives within the broader modelling landscape, derives their analytical properties, illustrates their behaviour through numerical examples, and discusses their relevance for fiscal and policy analysis in developing-country contexts, where transparency, interpretability and minimal data requirements are essential.

# Detailed Power Optimisation Models and their Representation In Principle in Reduced Form Models such as CPAT

Before introducing the penalty-based investment rules, it is useful to clarify how the underlying technological cost structure should be represented in a reduced-form model such as CPAT. In a realistic power system, the cost of deploying an additional unit of coal, gas, solar or wind is not simply its private levelised cost of electricity (LCOE). Instead it reflects a combination of energy production, reliability contributions and balancing requirements, all of which are jointly determined by the optimal dispatch of generators across the 8,760 hours of the year. Thus, any parsimonious modelling framework must begin with an understanding of how these system-level interactions translate into an effective marginal cost for each technology.

Consider a simplified generation expansion and dispatch model in which the planner determines hourly output qi,h and installed capacity Ki for each technology i to minimise total system cost subject to hourly load balance, capacity constraints and reserve or reliability requirements. Formally, the planner solves:

min{Ki,qi,h}i​FiKi+Vih=18760qi,h

subject to:

&i​qi,h=Lh  ∀h,&qi,h≤Ki⋅CFi,h  ∀i,h,&i​ELCCi(K1,…,Kn) Ki≥PRM⋅PeakLoad.

Here Fi and Vi are fixed and variable costs, CFi,h is the availability profile, and ELCCi denotes the effective load-carrying capability of technology i. The last constraint ensures that the combined capacity provides sufficient reliability given the peak-load requirement (PRM). This problem encompasses the basic operational and reliability features of modern power systems and captures interactions between variable renewables and firm capacity.

In this framework, the optimal solution produces not only the least-cost mix of capacities {Ki} but also a set of shadow prices on the constraints. These shadow prices have natural interpretations as system costs and system benefits. For example:

- The dual value on the hourly energy balance constraints indicates the marginal energy value of an additional unit of generation in each hour.

- The dual value on the firm-capacity (ELCC) constraint reflects the marginal value of reliability, which depends strongly on a technology’s load-carrying capability.

- The binding of ramping, curtailment or flexibility constraints yields a marginal value associated with flexibility, which typically favours gas and storage.

When the model is solved for multiple candidate technologies, these shadow prices can be combined into a virtual marginal cost or system-adjusted cost for each technology. For example, the effective marginal cost of a unit of solar capacity may be written as:

Csolarvirt=CsolarLCOE+γrel1−ELCCsolar+γflex⋅FlexReqsolar+γcur⋅Curtailmentsolar,

where each γ denotes the relevant shadow price from the optimisation and the terms represent, respectively, the capacity credit shortfall, flexibility burden and expected curtailment. Gas and coal would obtain corresponding adjustments, although the signs would differ: flexibility tends to be a benefit for gas, while firm capacity is a benefit for coal or nuclear.

A key point is that these virtual costs reflect the system-level consequences of adding marginal capacity of each technology—not the private LCOE alone. For a reduced-form model such as CPAT, this provides a natural way to translate complex 8,760-hour optimisation outcomes into a scalar system cost adder:

Cisys=Civirt−CiLCOE.

The total effective cost for technology i is then

Cieff=CiLCOE+Cisys.

In CPAT this effective marginal cost should be interpreted as a cost associated with the capacity share of each technology rather than the investment share. Once these system-adjusted costs are obtained (e.g. from a stylised or precomputed optimisation), they serve as the baseline cost input to the velocity and acceleration models introduced in the next section. In effect, the detailed optimisation maps into a reduced-form cost vector {Ci,teff} which captures the essential operational and reliability interactions.

The investment dynamics can then be interpreted as follows. In the absence of additional constraints, a rational investor would allocate all investment to the technology with the lowest Cieff.

Please see Appendix A for a mathematical specification of how an overall 8,760-hour optimisation model can specify a reduced-form one-time-period model before the introduction of velocity and acceleration.

Diversification pressures, adjustment frictions and political-economy constraints require smoothing of this allocation response. The velocity penalty introduces a systemic cost to excessive concentration, while the acceleration penalty limits rapid year-to-year changes. Together, these penalties allow CPAT to generate both a plausible long-run allocation based on effective system costs and realistic transition paths that respect operational and institutional constraints. The next sections introduce “clean” velocity and acceleration models and then introduce the combined system.

# A Velocity-Penalised Model of Investment Allocation

The first alternative considered in this paper is a static model in which the allocation of new investment across technologies is determined by a trade-off between relative cost competitiveness and an explicit penalty for excessive concentration. The term “velocity” is used in analogy with adjustment-cost models in macroeconomics: just as high adjustment speed incurs a penalty in capital accumulation models, here a high share of investment allocated to any one technology incurs a quadratic cost. This structure captures the intuition that technological, institutional, or system-integration factors may discourage placing a disproportionate share of new capacity in a single option, even if that option has the lowest levelised cost of electricity (LCOE). It also produces closed-form solutions that are transparent, intuitive, and compatible with the spreadsheet-based architecture of CPAT.

## Formulation

Suppose that a country must allocate one unit of new investment across N generation technologies indexed by i. Let si denote the share allocated to technology i, and let Ci represent its levelised cost. In a purely cost-driven model without frictions, least-cost selection would dictate that all investment flows to the lowest-cost option. In practice, however, countries rarely adopt such extreme strategies, either because diversification reduces risk, because grid integration concerns arise at high penetration rates, or because investment pipelines cannot be scaled arbitrarily fast.

To capture these considerations, we posit that the policymaker chooses investment shares {si} to minimise an augmented cost function:

i=1NCisi+λ2si2,

{#eq-velocity_objective} subject to the simplex constraint

i=1Nsi=1,  si≥0.

{#eq-simplex_constraint} The parameter λ>0 determines the severity of the diversification pressure. A large value of λ makes concentration costly and leads to more uniform investment shares, whereas a small value of λ makes the penalty negligible and approximates a least-cost allocation rule.

This formulation is closely related to the quadratic penalty structures used by @BauerHallegatteMcIsaac2025 in modelling political economy constraints on mitigation effort. It can also be interpreted as a reduced-form representation of system integration costs or institutional frictions that increase disproportionately as a single technology dominates the investment mix.

## Derivation of the closed-form allocation rule

To derive the optimal allocation, consider the Lagrangian

L(s1,…,sN;μ)=i=1NCisi+λ2si2−μi=1Nsi−1,

where μ is the multiplier associated with the constraint (@eq-simplex_constraint). The first-order conditions for an interior solution are

∂L∂si=Ci+λsi−μ=0,

which implies

si=μ−Ciλ.

{#eq-s_i_expression} Summing over i and enforcing the share constraint gives

i=1Nsi=1λNμ−i=1NCi=1,

leading to

μ=C+λN,

where C=1Ni=1NCi denotes the average cost across technologies. Substituting this back into (@eq-s_i_expression) yields the closed-form solution

si=1N+C−Ciλ.

{#eq-closedform_velocity}

Expression (@eq-closedform_velocity) reveals several desirable properties. First, the model naturally assigns higher shares to technologies with lower LCOE, with the magnitude of the response governed by 1/λ. Second, if a technology has a sufficiently high cost relative to others, the right-hand side becomes negative, causing the optimal share to be zero once the non-negativity constraint is enforced. Thus the model endogenously produces corner solutions in which dominated technologies receive no investment, addressing a key limitation of the logit rule. Third, the rule collapses to uniform diversification si=1/N as λ→∞, and to least-cost allocation (a corner solution) as λ→0.

## Boundary behaviour and zero-share conditions

The closed-form expression (@eq-closedform_velocity) represents the interior solution. However, if

1N+C−Ciλ<0,

then the non-negativity constraint binds and si=0. For such technologies, the remaining shares renormalise to satisfy the simplex constraint. Because the algebra is fully transparent, analysts can explicitly determine the cost threshold above which a technology is excluded. Let S=j≠i​Cj denote the sum of costs for all other technologies. Then the condition for technology i to retain a positive share can be expressed as

Ci<S+λN−1.

This threshold determines the maximum cost a technology may have while still contributing to the investment mix.

## Interpretation in terms of systemic LCOE

The model also has a compelling economic interpretation. Differentiating the objective function (@eq-velocity_objective) with respect to si yields an effective or “systemic” marginal cost

Cisystemic=Ci+λsi.

Thus the quadratic penalty can be understood as a technology-specific uplift applied in proportion to the investment share already allocated to that technology. In systems with high integration costs, for example, the systemic cost interpretation may provide a more meaningful representation of planners’ behaviour than either discrete-choice probabilities or optimisation duality. It also offers a natural way to incorporate diversification pressures without imposing exogenous constraints on maximum allowable shares.

## Calibration of the penalty parameter

Calibrating λ is central to applying this model in practice. If λ is too small, the model degenerates into a least-cost allocation rule, which may produce unrealistic concentration in a single technology. If λ is too large, the model forces a nearly uniform distribution that ignores economically meaningful cost differences. In CPAT-like contexts, where investment shares are updated annually and parameter transparency is important, one approach is to calibrate λ to match observed historical diversification patterns. Another approach is to choose λ such that a specified cost differential—for example, a 20 USD/MWh advantage— translates into a desired difference in investment shares. Because the formula (@eq-closedform_velocity) is linear in costs, such calibration is straightforward.

A further advantage is that the linear structure allows policymakers to inspect directly how results vary with λ. Sensitivity analysis can therefore be communicated in terms that are comprehensible to policymakers, who often prefer explicit trade-offs between diversification pressure and cost responsiveness rather than the probabilistic responses implied by logit models.

## Advantages relative to CPAT’s multinomial logit rule

Compared with the logit rule used in CPAT, the velocity-penalised model has several appealing features. It produces zero shares for highly uncompetitive technologies, avoids the obligatory interior solution of logit, offers direct economic interpretation via systemic LCOE, and provides a cost–share relationship that is both linear and transparent. Whereas CPAT’s logit rule ensures smoothness through an exponential function whose curvature may be unintuitive, the velocity model achieves smoothness by adjusting the effective costs in a way that is easy to explain to finance ministries.

# An Acceleration-Penalised Model of Transition Dynamics

The second alternative developed in this paper recognises that investment decisions are rarely made in isolation for a single year. Instead, they are embedded in multi-year planning cycles in which existing project pipelines, policy commitments, and administrative capacities constrain the pace at which a country can reconfigure its power-generation mix. For example, even if solar emerges as the lowest-cost technology, the availability of transmission capacity, the structure of public procurement, and the maturity of supply chains may limit the speed at which solar investment can be scaled up. Conversely, politically sensitive sectors such as coal or gas may not be reduced arbitrarily quickly because of employment concerns or regional development priorities. To capture these transition frictions, we introduce a model in which the investment decision is not penalised for the level of investment shares but for the change in shares over time. Whereas the velocity-penalised model produces an instantaneous steady-state allocation, the acceleration- penalised model governs the path taken to move from one allocation to another.

## A dynamic investment allocation problem

Let si,t denote the share of investment in technology i during year t. Let Ci,t denote the cost of that technology in year t, which may evolve due to learning, fuel prices, or policy. We assume that the planner minimises the one-step cost

Ci,tsi,t+λ2(si,t−si,t−1)2.

{#eq-acceleration_objective} The first term captures contemporaneous cost competitiveness, while the second term imposes a quadratic penalty on deviations from last year’s allocation. The penalty parameter λ>0 represents transition friction: large values of λ imply that the system can adjust only slowly, while small values allow rapid reallocation in response to cost signals.

Formally, at each time step the planner solves:

min{si,t}i​Ci,tsi,t+λ2(si,t−si,t−1)2,  s.t. i​si,t=1, si,t≥0.

This problem resembles dynamic adjustment-cost models in investment theory, where the cost of changing the capital stock introduces inertia. Here, inertia applies to investment shares rather than capacities, making the formulation suitable for spreadsheet environments and for annual policy simulations such as those performed in CPAT.

## Derivation of the update rule

To derive the first-order condition, consider the Lagrangian

Lt=i​Ci,tsi,t+λ2(si,t−si,t−1)2−μti​si,t−1,

where μt enforces the share constraint in period t.

Differentiating with respect to si,t yields:

Ci,t+λ(si,t−si,t−1)−μt=0,

{#eq-acceleration_foc} so that

si,t=si,t−1+μt−Ci,tλ.

{#eq-acceleration_update}

Equation (@eq-acceleration_update) provides an elegant and transparent update rule. The share of technology i adjusts upward if its cost is below the shadow price μt and downward if its cost is above it. The factor 1/λ determines the sensitivity of adjustment.

The multiplier μt ensures the shares sum to one. Summing (@eq-acceleration_update) over all i gives:

1=1+Nμt−i​Ci,tλ,

implying

μt=1Ni​Ci,t.

Thus, as in the velocity model, the shadow price equals the average cost across technologies.

Substituting into (@eq-acceleration_update) yields the final update rule:

si,t=si,t−1+Ct−Ci,tλ.

{#eq-s_final_acceleration}

This rule is simple to implement and interpret: technologies that become cheaper than the current average cost gain investment share, and those that become more expensive lose share, with the speed of adjustment governed by λ.

## Interpretation via Euler–Lagrange dynamics

The update rule (@eq-s_final_acceleration) admits an interpretation analogous to Euler– Lagrange equations in continuous-time optimisation. If time were continuous and costs varied smoothly, one would obtain a differential equation of the form:

λdsidt=C(t)−Ci(t).

In this interpretation, the cost gradient acts as a “force” pushing the system toward more cost-effective technologies, while the parameter λ acts as a frictional term that dampens the response. Although our interest is in discrete annual steps, the analogy clarifies the interpretation: the acceleration model moderates the velocity of change in the investment mix.

## Steady-state properties

Unlike the velocity model, the acceleration model does not yield an interior steady-state share distribution unless all technologies have identical costs. From (@eq-s_final_acceleration), a steady state si,t=si,t−1 would require

Ci,t=Ct for all i,

which occurs only in degenerate cases. In practice, the model smoothly transitions toward a corner solution determined by cost differences. This behaviour closely resembles dynamic versions of the multinomial logit rule, where shares evolve gradually even if the long-run allocation collapses to the least-cost option.

In many policy contexts, the transition path is more relevant than the asymptotic distribution. Governments often commit to long-term technology targets but are constrained by the pace at which investment can realistically be shifted. The acceleration model provides a natural means of representing such constraints.

## Handling boundary constraints

If equation (@eq-s_final_acceleration) yields a negative value for si,t, the share is set to zero and the remaining shares are renormalised. The model therefore permits both gradual withdrawal from high-cost technologies and re-entry should costs fall in future years. This feature is particularly useful for modelling gas, coal, or emerging technologies such as green hydrogen, whose competitiveness can vary substantially over time.

## Relation to evolutionary diffusion models

The structure of (@eq-s_final_acceleration) also bears a conceptual similarity to the evolutionary diffusion equation underlying FTT:Power [@Mercure2012]. Both frameworks govern the rate of change of market shares rather than the instantaneous level, and both embed a cost gradient in the direction of substitution. However, the acceleration model is substantially simpler: it is linear rather than logistic, does not require pairwise substitution probabilities, and produces easily interpretable parameters. Whereas FTT requires substantial parameterisation and calibration, the acceleration model can be implemented in Excel with a single free parameter.

## Calibration of transition speed

The penalty parameter λ determines the responsiveness of the investment mix. A large value of λ implies gradual adjustment, reflecting institutional, technical, or financial constraints on reallocation. A small value implies rapid shifts, suitable for scenarios with strong policy support, abundant administrative capacity, or highly modular technologies such as solar.

Calibration may be based on historical rates of change in technology investment shares, on plausible annual build rates for relevant technologies, or on political considerations such as the feasibility of phasing down coal over a prescribed period. Because the model is linear and the parameter has direct mechanical interpretation, calibration can be communicated clearly to policymakers.

## Advantages and limitations

The acceleration-penalised model provides a parsimonious and intuitive method for shaping transition pathways. It is particularly well suited for LTS, NDC updates, or CPAT analyses where a policymaker wants to understand how quickly the investment mix may be able to shift under different assumptions about institutional constraints. Its principal limitation is that it does not provide a meaningful long-run equilibrium unless costs converge. For equilibrium analysis, the velocity model of Section 2 is therefore preferred. In practice, both models can be used jointly: the velocity model to determine a cost-consistent long-run allocation and the acceleration model to govern the pace of movement toward it.

# Numerical Illustrations

To illustrate the behaviour of the two penalty-based allocation models, this section presents a stylised numerical example involving five representative power-generation technologies. The example is not intended to reproduce any particular national context; rather, its purpose is to clarify the mechanics of the allocation rules and show how they differ from logit-based and optimisation-based approaches. All numerical values are chosen for pedagogical clarity.

## Technology set and baseline costs

Consider the following set of technologies and levelised costs, expressed in USD/MWh:

| Technology | Ci (USD/MWh) |
| --- | --- |
| Solar | 20 |
| Gas | 30 |
| Coal | 40 |
| Wind | 50 |
| Marine | 100 |

The ordering reflects a common pattern in many emerging markets: solar is highly competitive, gas is moderately priced, coal and wind are somewhat more expensive, and marine technologies remain prohibitively costly for near-term deployment.

Throughout this section, we consider two values of the penalty parameter λ:

- λ=100, representing moderate diversification pressure,

- λ=200, representing stronger diversification pressure.

These values are chosen for illustration and do not reflect a specific empirical calibration.

## Steady-state allocation under the velocity model

Using the closed-form velocity allocation rule derived in Section 2,

si=1N+C−Ciλ,

{#eq-velocity_rule_repeat} we obtain steady-state shares for the five technologies.

The average cost is:

C=20+30+40+50+1005=48.

Substituting values into (@eq-velocity_rule_repeat) with λ=100 yields:

ssolar=0.20+48−20100=0.48,

sgas=0.20+48−30100=0.38,

scoal=0.20+48−40100=0.28,

swind=0.20+48−50100=0.18,

smarine=0.20+48−100100=−0.32.

Because negative shares violate the non-negativity constraint, the marine share is set to zero and the remaining shares are renormalised to sum to one. The resulting investment mix is:

| Technology | Share (%) |
| --- | --- |
| Solar | 36.4 |
| Gas | 28.8 |
| Coal | 21.2 |
| Wind | 13.6 |
| Marine | 0.0 |

Two observations follow immediately. First, the velocity model naturally eliminates marine generation, reflecting its extremely high cost. Second, despite solar being the least-cost option, it does not absorb all investment; instead, diversification arises endogenously through the penalty term. This stands in contrast to least-cost optimisation models, which would place all investment in solar absent additional constraints, and to logit models, which would force marine to retain a positive share even when its cost is uncompetitive.

## Sensitivity to the diversification penalty

If we increase the penalty to λ=200, cost differentials exert weaker influence. The closed-form expression becomes:

si=0.20+C−Ci200.

Substituting values yields:

ssolar=0.20+28200=0.34,

sgas=0.20+18200=0.29,

scoal=0.20+8200=0.24,

swind=0.20−2200=0.19,

smarine=0.20−52200=−0.06.

After clipping and renormalisation, marine is again excluded. The remaining technologies show greater uniformity than in the λ=100 case. This illustrates a general property: as λ increases, the system becomes less sensitive to cost differences and more evenly diversified.

## Cost threshold for participation

The velocity model provides an analytic condition for determining whether a technology receives a positive share. A technology i retains a positive share if:

Ci<S+λN−1,

{#eq-threshold_condition_repeat} where S is the sum of costs of all other technologies.

For marine generation,

S=20+30+40+50=140.

The threshold for participation is therefore:

Cmarinemax=140+1004=60,

when λ=100. Because the actual cost of marine is 100 USD/MWh, the model assigns it a zero share. This is consistent with the intuition that no amount of diversification pressure can justify investment in a technology that is so far outside the competitive range.

If λ were increased to 300, the threshold would rise to:

Cmarinemax=140+3004=110,

making marine marginally admissible. This illustrates how the diversification parameter shapes the model’s willingness to tolerate expensive technologies.

## Transition dynamics under the acceleration model

To illustrate how the acceleration-penalised model governs transitions, consider again the cost vector (20,30,40,50,100). Suppose the system begins with an equal allocation:

si,0=0.20 for all i.

Using the update rule derived in Section 3,

si,t=si,t−1+C−Ciλ,

and setting λ=100, the first-step adjustments are:

ssolar,1=0.20+28100=0.48,

sgas,1=0.20+18100=0.38,

scoal,1=0.20+8100=0.28,

swind,1=0.20−2100=0.18,

smarine,1=0.20−52100=−0.32.

After clipping, the transition mirrors exactly the velocity allocation in the first period. In subsequent years, the system continues to shift share toward solar and gas until marine’s share stabilises at zero. For values of λ larger than 100, the transition occurs more slowly and with more gradual year-to-year adjustments.

An important distinction emerges: under the velocity model, the above distribution is the steady-state allocation; under the acceleration model, the system continues to evolve beyond the first period unless cost differentials vanish. Thus the acceleration model is better interpreted as a pathway generator rather than a steady-state predictor.

## Policy interpretation

These numerical examples underline several implications for policy modelling. First, the velocity model provides an analytically clean and intuitive formula for the distribution of investment in a hypothetical steady state given relative costs and diversification pressure. Second, the acceleration model provides a mechanism for exploring realistic short- and medium-run pathways, especially in contexts where political or administrative constraints limit the pace of change. Third, the analytic threshold for zero shares is particularly valuable in fiscal policy contexts, where the objective is often to illustrate how taxes or subsidies can render technologies marginal or obsolete.

Both models are sufficiently transparent that their behaviour can be explained directly to finance ministries, which considerably enhances their usefulness in tools such as CPAT.

# Land-use-related system costs and full effective cost

In addition to integration and flexibility effects, large-scale deployment of variable renewables is constrained by available land. We introduce a simple land-use-related system cost that grows quadratically with the total land fraction occupied by solar and wind, and is already noticeable when renewables occupy about 2.5% of the country’s land area.

Let Atot denote total land area of the country (m2). For solar PV, assume a peak power density of 100 Wpeak/m2 and a capacity factor of 20%. For wind, assume 16 Wpeak/m2 and a capacity factor of 25%. The capacity–area relationships (in MW) are then:

Ksol,t&=100 Asol,t106=Asol,t104,  ⇒ Asol,t=104 Ksol,t,Kwin,t&=16 Awin,t106=Awin,t6.25×104,  ⇒ Awin,t=6.25×104 Kwin,t,

where Asol,t and Awin,t are the land areas used by solar and wind, respectively.

The total land area used by these renewables in year t is

ARES,t=Asol,t+Awin,t=104Ksol,t+6.25×104Kwin,t,

and the corresponding land-use fraction is

ft=ARES,tAtot.

We assign the land-use penalty primarily to solar, with the cost scaling quadratically in the total land fraction ft. To make the effect noticeable, we choose the coefficient so that the land-use cost for solar reaches approximately $10/MWh when renewables occupy 2.5% of total land area:

Csol,tland=ζsolft2,  ζsol=100.0252≈1.6×104 USD/MWh.

Thus, at ft=0.025,

Csol,tland≈10 USD/MWh,

and the penalty grows rapidly as the land fraction increases. For wind, an analogous but smaller coefficient ζwin can be used if desired; for simplicity, it may be set to zero or a fraction of ζsol.

Collecting all components, the full effective cost per MWh that drives the investment rule in year t can be written as

Ci,teff,full=Ci,tbase+Ci,tsys(st−1;D)+Ci,tland(ft)+λvsi,t+λa(si,t−si,t−1),

{#eq-full_effective_cost_land} where:

- Ci,tbase is the base LCOE;

- Ci,tsys(st−1;D) captures integration, flexibility and other system effects (e.g. gas flexibility benefits, renewable balancing costs, density-related constraints);

- Ci,tland(ft) is the land-use penalty defined above, significant when the combined solar–wind land fraction ft reaches a few percent of total area;

- λvsi,t and λa(si,t−si,t−1) are the marginal contributions of the velocity and acceleration penalties, respectively, arising from the quadratic investment objective.

The velocity–acceleration optimality conditions are obtained as in the main text, replacing Ci,teff with Ci,teff,full in the first-order conditions and in the closed-form investment share rule.

# CPAT Implementation Recommendations and Combined Specification

This section offers concrete recommendations for implementing the proposed allocation models in CPAT and presents a combined specification that incorporates both velocity and acceleration penalties alongside simple representations of system benefits and costs. The aim is to retain spreadsheet-level simplicity while capturing the most important features of realistic power system transitions.

## Recommended structure for CPAT

For CPAT and similar fiscal tools, it is natural to use the velocity-penalised model as the backbone of the investment allocation module. The closed-form steady-state solution is transparent, easy to explain to finance ministries, and provides a direct link between relative costs, diversification pressure and the exclusion of high-cost technologies. This makes it well suited for the primary counterfactual comparisons that CPAT performs across tax and subsidy scenarios.

At the same time, it is often useful to moderate the speed at which investment shares respond to policy-induced cost changes. For example, a sudden increase in carbon taxation might, in principle, justify an immediate shift away from coal, but in practice administrative, technical and political constraints will slow the response. For this reason, CPAT can include a weak acceleration penalty that constrains year-to-year changes in investment shares without dominating the allocation outcome. In practical terms, this means choosing the velocity penalty parameter to govern the long-run distribution of investment and setting the acceleration penalty parameter to a relatively small value that primarily affects short-run dynamics.

## A combined velocity–acceleration model

Let i index technologies and t index years. Denote by si,t the share of new investment in technology i in year t. For CPAT implementation we propose the following generic one-period problem:

min{si,t}i​Ci,tbasesi,t+Ci,tsys(st−1;D)si,t+λv2si,t2+λa2(si,t−si,t−1)2,

{#eq-combined_objective} subject to

i​si,t=1,  si,t≥0.

Here, Ci,tbase is the base levelised cost of electricity for technology i in year t (as already computed in CPAT), while Ci,tsys(st−1;D) represents a set of system-level adjustments evaluated on the previous year’s shares st−1 and a country-level measure of population density D. Evaluating system adjustments on lagged shares keeps the problem linear-quadratic in st and preserves analytical tractability.

In line with CPAT’s focus on a small number of interpretable terms, we propose the following decomposition of Ci,tsys(st−1;D):

Ci,tsys(st−1;D)=−βGGisgas,t−1+κRRiSRES,t−1+ϕiDsi,t−1,

{#eq-C_sys_definition} where:

- Gi is an indicator equal to one if technology i is gas, and zero otherwise; the term −βGGisgas,t−1 captures the system benefit of flexible gas capacity, which increases as the gas share rises, effectively lowering its marginal cost;

- Ri is an indicator for variable renewable technologies (such as solar and wind); the term κRRiSRES,t−1 captures additional system costs associated with storage, balancing and grid integration that rise with the aggregate share of variable renewables

SRES,t−1=j​Rjsj,t−1;

- ϕi is a technology-specific parameter scaling a quadratic physical constraint linked to population density; the term (ϕi/D)si,t−1 represents the idea that, for a given technology, the effective marginal system cost rises with its local spatial footprint when population density is low (for example, for land-intensive renewables in sparsely populated countries).

Substituting (@eq-C_sys_definition) into (@eq-combined_objective) and noting that the system adjustments depend only on lagged shares, the objective remains quadratic in si,t. The Lagrangian for year t can therefore be written as

Lt&=i​Ci,tbase+Ci,tsys(st−1;D)si,t+λv2si,t2+λa2(si,t−si,t−1)2& −μti​si,t−1.

The first-order condition with respect to si,t is

Ci,tbase+Ci,tsys(st−1;D)+λvsi,t+λa(si,t−si,t−1)−μt=0.

Collecting terms in si,t yields:

(λv+λa)si,t=μt−Ci,tbase−Ci,tsys(st−1;D)+λasi,t−1.

{#eq-combined_foc} Thus the optimal share in year t is given by

si,t=1λv+λaμt−Ci,tbase−Ci,tsys(st−1;D)+λasi,t−1.

{#eq-combined_update_pre_mu}

The multiplier μt is determined by the share constraint. Summing (@eq-combined_update_pre_mu) over i and imposing i​si,t=1 gives:

1=Nμt−i​Ci,tbase−i​Ci,tsys(st−1;D)+λai​si,t−1λv+λa.

Using i​si,t−1=1, this simplifies to

μt=Ctbase+Ctsys(st−1;D)−λaN+λv+λaN,

where

Ctbase=1Ni​Ci,tbase,  Ctsys(st−1;D)=1Ni​Ci,tsys(st−1;D).

It is convenient to define an adjusted average cost

Ctadj=Ctbase+Ctsys(st−1;D)−λaN.

Then the multiplier becomes

μt=Ctadj+λv+λaN.

Substituting this back into (@eq-combined_update_pre_mu) yields a closed-form update rule:

si,t=1N+Ctadj−Ci,tbase−Ci,tsys(st−1;D)+λasi,t−1λv+λa.

{#eq-combined_update_final}

Equation (@eq-combined_update_final) is the central expression for CPAT implementation. It states that each technology’s share is the sum of (i) an equal-share baseline 1/N and (ii) an adjustment term that depends on the difference between adjusted average cost and its own effective cost, plus a contribution from last year’s share scaled by the acceleration parameter. When λa is set to zero and the system cost adjustments are dropped, this equation collapses to the pure velocity model. When λv is large and λa is small, the model behaves like a lightly smoothed version of least-cost allocation with system adjustments.

The total effective cost of investing in technology i in year t can be written compactly as:

Ci,ttot(st,st−1;D)=Ci,tbase+Ci,tsys(st−1;D)+λvsi,t+λa(si,t−si,t−1),

{#eq-total_LCOE} which clarifies the sense in which velocity and acceleration penalties adjust the levelised cost to reflect diversification pressures and transition frictions.

## Calibration Strategy

A practical calibration strategy for CPAT can proceed in stages. First, the system adjustment parameters (βG,κR,ϕi) can be informed by engineering studies and existing estimates of balancing costs, storage requirements and grid expansion needs. These estimates can be translated into approximate marginal cost adjustments per unit of gas flexibility, per unit of variable renewable share and per unit of technological footprint scaled by population density.

Second, the velocity and acceleration parameters (λv,λa) can be calibrated to historical data on investment shares and costs. Specifically, one can collect a panel of {Ci,tbase,si,t} for a country or peer group and estimate λv and λa so that the model-implied shares given by (@eq-combined_update_final) approximate observed shares. A simple approach is to minimise the sum of squared deviations between observed and modelled shares over a historical window, possibly using regularisation to ensure that λa remains small relative to λv.

Finally, calibration results should be cross-checked for plausibility in policy dialogue. If the estimated λv implies almost uniform diversification regardless of costs, or if λa implies unrealistically slow transitions, parameters can be adjusted within ranges consistent with historical experience and stakeholder judgement. Because the parameters have direct mechanical interpretation, these adjustments can be communicated explicitly, which is an advantage relative to more opaque functional forms.

## Integration into CPAT

CPAT requires a simple, robust, and transparent method for determining investment responses to policy-induced cost changes. The existing multinomial logit rule provides smoothness and ease of calibration but also forces positive shares for all technologies. The velocity model can replace or complement this rule in applications where it is important to illustrate the complete elimination of high-cost technologies or to ensure that investment shares reflect plausible diversification patterns.

The acceleration model plays a distinct role. In many country engagements, the central question is not which technologies ultimately dominate, but how quickly the system can transition. CPAT scenarios that incorporate carbon taxation, fossil-fuel subsidy reform, or renewable energy support policies can benefit from modelling the pace of reallocation rather than relying solely on instantaneous responses. The acceleration model provides this capability while preserving the interpretability required for communication with finance ministries.

Both models can be integrated into CPAT in a modular fashion. The velocity model defines a steady-state target allocation for each policy scenario. The acceleration model can then govern the multi-year path toward that target. This combination mirrors common practice in long-term strategy (LTS) planning, where a country commits to a 2050 mix and then explores feasible transition pathways.

# Discussion

The preceding sections developed two analytically tractable alternatives to the multinomial logit investment rule widely used in policy simulation tools such as CPAT. The velocity-penalised model produces a closed-form steady-state distribution of investment shares that depends on relative costs and a diversification parameter. The acceleration-penalised model governs the pace of change in the investment mix, yielding smooth transitions consistent with institutional and political constraints. This section situates these approaches within the broader modelling landscape and outlines their implications for fiscal policy and long-term planning.

## Comparison with existing modelling paradigms

Least-cost optimisation models such as TIMES represent the most rigorous and detailed approach to power-sector investment planning. Their strength lies in the ability to encode a wide range of technical, network, and resource constraints. However, their computational requirements render them unsuitable for country-level fiscal diagnostics, where policymakers require rapid scenario assessment in a spreadsheet environment. Moreover, least-cost optimisation tends to produce corner solutions unless diversification constraints are explicitly introduced. The velocity model developed here embeds diversification directly in the objective function, achieving a similar effect with minimal computational overhead.

LEAP-style accounting models offer transparency and accessibility, but they lack explicit behavioural rules determining investment allocation. Their strength lies in narrative scenario construction rather than equilibrium modelling. The penalty-based models presented here provide a behavioural closure that remains consistent with LEAP-style narrative pathways and can be embedded directly in tools designed for policy communication.

FTT diffusion models offer a rich representation of technological substitution driven by pairwise comparisons and learning-by-doing. They capture momentum effects, lock-in, and non-linear dynamics, but they require large parameter sets and careful calibration. The acceleration model shares with FTT the focus on transition dynamics, but its linear structure and single-parameter governance make it much more suitable for fiscal policy tools. In contrast to logit-based models, the penalty models allow a technology to drop out entirely when it is prohibitively costly.

## Macro-fiscal implications and MFMod integration

From a fiscal perspective, investment allocation affects public revenues and expenditures through several channels: electricity tariffs, fossil-fuel tax bases, subsidy requirements, public investment obligations, and external financing needs. MFMod, the World Bank’s standard macro-fiscal model, links sectoral investment to macroeconomic aggregates, allowing analysts to quantify how different technology pathways affect debt sustainability, GDP growth, and fiscal space.

The penalty-based approaches described here provide a lightweight but disciplined behavioural rule that can be fed into MFMod. For example, an acceleration-constrained scenario in which renewable energy uptake proceeds slowly may imply a prolonged period of fossil-fuel tax revenue or sustained fuel import bills. In contrast, a velocity-based allocation with strong diversification pressure may shift investment more rapidly toward low-cost technologies, reducing the fiscal burden of subsidies or grid expansion.

Because the penalty parameters are interpretable, they can be mapped to institutional realities: permitting timelines, grid bottlenecks, procurement cycles, or political feasibility constraints. This mapping enables transparent and policy-relevant macro-fiscal scenarios.

## Relevance for long-term strategies and NDC updates

Long-term strategies (LTS) and enhanced NDCs require countries to articulate realistic and credible transition pathways. In these contexts, the penalty models offer several advantages. The velocity model clarifies the cost context in which diversification or technology neutrality may be desirable. The acceleration model allows analysts to constrain the pace of coal phase-out or renewable scale-up in ways consistent with administrative and financial capacity.

For developing countries, where rapid transitions may be limited by fiscal constraints, these models allow a direct representation of the trade-off between ambition and feasibility. They also provide a principled means to evaluate the impact of international support, such as concessional finance or carbon contracts for difference (CCfDs), on the speed and direction of investment flows.

## Limitations and avenues for extension

The penalty-based models deliberately abstract from several important features of real-world power systems. They do not incorporate learning curves, resource depletion, reliability constraints, grid integration costs, or storage requirements. Nor do they consider feed-in constraints or investment indivisibilities. These features can be introduced in future extensions by modifying the cost term or by adding constraints to the objective function.

The linearity of the acceleration model is both a strength and a limitation. It enables clear interpretation but does not capture the logistic behaviour observed in historical technology diffusion. Hybrid models that combine linear penalties with logit-like diffusion functions may offer a pragmatic compromise between realism and tractability.

# Recommendations

The paper has the following two main recommendations for CPAT.

First, adopt the velocity rule above as the main capacity constraint, replacing the renewable energy constraint. Include a minor acceleration term as well, so the overall equation includes systems cost, physical constraints, velocity and acceleration. Use the velocity rule instead of the existing multilogit plus RE constraint.

Second, consider a better approach to using systems costs to determine least-cost optimised levels of capacity additions. Consider updating the investment and retirement rules to better reflect what those optimised levels of capacity are. The appendix modifies the CPAT approach in more detail, integrated with the velocity-acceleration rule.

# Conclusion

This paper developed two analytically tractable alternatives to the multinomial logit rule commonly used in power-sector investment modelling. The velocity-penalised model yields a closed-form steady-state distribution of investment shares driven by relative costs and a diversification parameter. The acceleration-penalised model governs the pace of transition, capturing institutional and political constraints.

Both models are simple enough to be implemented in spreadsheets while retaining sufficient structure for integration into CPAT and macro-fiscal tools such as MFMod. Their transparency makes them particularly suitable for engagement with finance ministries, where clarity and interpretability are essential. By providing both steady-state and dynamic behaviour, the models support long-term planning, fiscal risk analysis, and the design of realistic policy pathways.

Future work may extend these models to include learning dynamics, resource depletion, explicit grid constraints, or hybrid functional forms blending diversification penalties with diffusion-based approaches. Nonetheless, even in their present form, the models offer a valuable addition to the analytical toolkit for climate and fiscal policy.

# Acknowledgements

To be added later. Paper written with substantial input from Florent McIsaac.

## Declaration of Interests

The author declares no conflicts of interest.

## Data Availability

No new data were created for this study. All modelling examples use stylised inputs.

# Yearly Reduced Form of Least-Cost Hourly Optimisation Models without Velocity or Acceleration Penalties

## Static least-cost benchmark and shadow values

Consider a stylised least-cost capacity and dispatch problem over 8,760 hours. The planner chooses capacities Ki and hourly outputs qi,h to minimise annualised fixed and variable costs:

min{Ki,qi,h} &i​FiKi + h=18760i​Viqi,hs.t. &i​qi,h=Lh,&&∀h,&0≤qi,h≤KiCFi,h,&&∀i,h,&i​ELCCiKi ≥ PRM⋅PeakLoad.&&

Let λh denote the dual variables on the hourly balance constraints (energy shadow prices), ϕi,h≥0 the duals on the upper bounds qi,h≤KiCFi,h (capacity scarcity in each hour), and μ≥0 the dual on the reliability constraint. The Lagrangian is

L = &i​FiKi+h,i​Viqi,h+h​λh​Lh−i​qi,h&+i,h​ϕi,h​KiCFi,h−qi,h+μ​i​ELCCiKi−PRM⋅PeakLoad.

The first-order conditions for qi,h and Ki yield, for technologies that are marginal in the optimum,

Vi−λh−ϕi,h&=0,  &&(marginal energy cost)Fi+h​ϕi,hCFi,h+μ ELCCi&=0,&&(marginal capacity cost).

The solution of this static problem defines: (i) an optimal capacity vector Ki⋆, and (ii) an effective annualised cost of capacity for each technology,

Cieff ≡ Fi+h​ϕi,hCFi,h+μ ELCCi,

which incorporates both system-integration and reliability burdens via the shadow prices ϕi,h and μ. In the reduced-form annual model below, we treat {Ki⋆,Cieff} as exogenous outputs of the fully optimising benchmark.

## One-period capital updating rule

In the annual reduced-form model we no longer resolve hourly dispatch. Instead, we track installed capacities Kit at the start of year t, exogenous retirements Rit (normal end-of-life), and new investment Iit. Capital evolves as

Kit+1=Kit−Rit+Iit,

with no early retirements: only Rit leaves the system.

Let net available capacity after retirements be Kit≡Kit−Rit, and suppose demand plus the planning reserve margin in year t​+​1 requires total firm capacity Kreqt+1. The additional firm capacity needed is

ΔKreqt = max0, Kreqt+1−i​ELCCiKit.

We choose non-negative investments Iit to meet exactly this requirement:

i​ELCCiIit=ΔKreqt,  Iit≥0.

To avoid over-investing in technologies that are already above their static least-cost level, we define the remaining “gap” to the benchmark capacity:

gapit≡max{0,Ki⋆−Kit}.

New investment is then allocated in two steps:

(i) Move toward the least-cost mix. If the required addition is smaller than the total remaining gap,

ΔKreqt≤j​ELCCj gapjt,

we distribute investment proportionally to these gaps (in firm-capacity terms):

Iit = ΔKreqt ELCCi gapitj​ELCCj gapjt.

In this case all new capacity moves the system strictly closer to the static optimum Ki⋆, and no technology overshoots its benchmark.

(ii) Fill any remaining requirement at least cost. If instead

ΔKreqt>j​ELCCj gapjt,

we first close all gaps (Iit≥gapit where needed), and allocate the remaining required firm capacity ΔKreqt−j​ELCCj gapjt by minimising the effective cost of new build:

min{Iit} i​CieffIit s.t. i​ELCCiIit=ΔKreqt−j​ELCCj gapjt,  Iit≥0.

The solution is a simple merit-order rule: any remaining capacity beyond Ki⋆ is built in the technologies with the lowest Cieff.

Under this updating rule, the annual model always: (i) installs just enough new capacity to meet demand (plus reserves), (ii) moves monotonically toward the least-cost capacity vector Ki⋆ whenever possible, and (iii) uses the shadow-price-based effective costs Cieff to govern which technologies expand when the system is already close to the static optimum.

## Early retirement as an extension

We now extend the annual model to allow for early retirement of existing capacity, in addition to exogenous end-of-life retirements. Let Eit≥0 denote early retirements of technology i in year t. Capital then evolves as

Kit+1=Kit−Rit−Eit+Iit,

where Rit are normal retirements and Iit are new investments. We continue to rule out negative investment or re-entry: Eit≥0, Iit≥0.

For each technology i, let Cikeep denote the annualised cost of keeping one unit of existing capacity online for one more year (including fixed O&M and expected operating costs valued at the optimiser’s prices). Let Cinew denote the effective annualised cost of building one unit of new capacity, as defined in the static benchmark above: Cinew≡Cieff.

Given demand and a planning reserve margin in year t​+​1, total firm capacity must again satisfy

i​ELCCiKit+1 ≥ Kreqt+1.

Substituting the capital-update equation and rearranging yields a requirement on the net effect of early retirements and new investment:

i​ELCCi−Eit+Iit ≥ ΔKreqt,

where

ΔKreqt ≡ Kreqt+1−i​ELCCi(Kit−Rit)

is the net additional firm capacity required relative to the post-normal-retirement stock.

Taking the static capacities Kit as given, the annual planner chooses early retirements and new investments to minimise the cost of meeting this firm-capacity requirement:

min{Eit,Iit} i​Cikeep(Kit−Rit−Eit)+i​CinewIit s.t. i​ELCCi(−Eit+Iit)≥ΔKreqt, Eit,Iit≥0.

Dropping constant terms that do not depend on Eit or Iit, this is equivalent to

min{Eit,Iit} i​− CikeepEit+CinewIit s.t. i​ELCCi(−Eit+Iit)≥ΔKreqt.

Let λt≥0 be the shadow price of the firm-capacity requirement. The Lagrangian is

Lt=i​− CikeepEit+CinewIit+λtΔKreqt−i​ELCCi(−Eit+Iit).

For technologies at an interior solution (Eit>0 or Iit>0), the first-order conditions are

∂Lt∂Eit&=− Cikeep+λtELCCi=0 ⇒ λt=CikeepELCCi,∂Lt∂Iit&=Cinew−λtELCCi=0 ⇒ λt=CinewELCCi.

Thus, in optimum, the planner compares the cost per unit of firm capacity of keeping versus replacing:

Cikeep ≡ CikeepELCCi,  Cinew ≡ CinewELCCi.

The optimal early-retirement rule can be stated simply as:

Retire early any capacity for which Cikeep>minjCjnew,

i.e. it is cheaper (per unit of firm capacity) to replace that plant with the lowest-cost available new technology than to keep it online.

Once optimal early retirements Eit have been determined using this rule, the remaining capacity Kit=Kit−Rit−Eit defines a new net requirement

ΔKreqt,post=max0, Kreqt+1−i​ELCCiKit,

and the new investment Iit is then allocated exactly as in the previous subsection, using the same least-cost rule based on Cinew=Cieff. Early retirement therefore operates as a preliminary screening step, potentially increasing the required volume of new investment but leaving the merit order over technologies unchanged.

# System Costs, Optimal Capacities, and an Adjusted Velocity Rule

In CPAT at present new investment is allocated according to the modified (multilogit) least-cost approach. But this takes no account if existing capacity is optimal or not.

In this note we sketch how information on marginal system costs as a function of capacity can be used to adjust the velocity-based investment rule. The objective is deliberately simple: we first characterise an “optimal” capacity mix implied by system costs alone, and then show how the velocity rule can be modified so that investment shares respond not only to levelised costs but also to the gap between actual and optimal capacities. Later then we formulate the approach without access to the “ideal” capacities, only on the systems costs.

## Optimal capacities from marginal system costs

Let Ki denote the installed capacity of technology i (in MW, or normalised units), and let MSCi(Ki) denote its marginal system cost as a function of capacity. This marginal cost may be thought of as the derivative of a system cost function

SCi(Ki)=0KiMSCi(x) dx,

which aggregates, for example, integration costs, balancing costs, network costs and any capacity-related system benefits.

In a purely system-cost-driven optimum (holding demand fixed and abstracting from levelised generation costs), the capacity mix {Ki*} satisfies the usual first-order condition:

MSCi(Ki*)=μsys for all technologies i with Ki*>0,

{#eq-MSC_opt_condition} where μsys is the common marginal system cost at the optimum. Equation (@eq-MSC_opt_condition) simply states that, at the system optimum, the marginal system cost of expanding any active technology is equalised.

We do not need to solve explicitly for Ki* in CPAT. It is sufficient to assume that (from engineering studies or a separate calculation) a benchmark capacity vector Ki* consistent with (@eq-MSC_opt_condition) is available, together with approximate slopes

mi=d MSCidKi|Ki*,

which describe how marginal system cost increases (or decreases) when capacity deviates from its benchmark.

## A simple investment problem with capacity-gap adjustments

Let si denote the share of new investment going to technology i in the current year, with i​si=1. We now define a deliberately simplified myopic investment problem:

min{si}i​Cisi+qi(Ki−Ki*)si+λ2si2 s.t. i​si=1, si≥0,

{#eq-capacity_gap_objective} where Ci is the base (private) LCOE, λ>0 is the familiar velocity penalty parameter, and qi is a parameter that links the capacity gap (Ki−Ki*) to an incremental cost or benefit in the investment decision.

Intuitively, if Ki>Ki* and qi>0, then the term qi(Ki−Ki*)si raises the effective cost of investing further in technology i, reflecting the fact that its capacity is already above the system optimum. Conversely, if Ki<Ki*, the same term reduces the effective cost, encouraging investment to close the gap.

## Lagrangian and adjusted velocity rule

The Lagrangian for (@eq-capacity_gap_objective) is

L(s,μ)=i​Cisi+qi(Ki−Ki*)si+λ2si2−μi​si−1,

where μ is the multiplier on the share constraint. The first-order condition for an interior solution is

Ci+qi(Ki−Ki*)+λsi−μ=0.

Define an “effective” cost

Ci=Ci+qi(Ki−Ki*),

{#eq-effective_cost_def} which incorporates both the base LCOE and the capacity-gap adjustment. Then the first-order condition simplifies to

Ci+λsi−μ=0,

so that

si=μ−Ciλ.

Summing over i and using i​si=1 yields

1=Nμ−i​Ciλ,

so that

μ=C+λN,  C=1Ni​Ci.

Substituting back, we obtain the adjusted velocity rule:

si=1N+C−Ciλ,

{#eq-adjusted_velocity_rule} with Ci defined in (@eq-effective_cost_def). The non-negativity constraint is handled as before by clipping negative shares to zero and renormalising.

In words, the optimal investment rule remains a velocity-style linear mapping from costs to shares, but the relevant cost is now an effective cost that reflects both the base LCOE and the deviation of capacity from its system-optimal level. If Ki>Ki*, the effective cost is higher, and si is smaller than in the pure velocity model. If Ki<Ki*, the effective cost is lower, and si is larger.

## Capacity dynamics and steady state

To close the loop, we need a simple mapping from investment shares to capacity. A deliberately oversimplified choice is

Ki,t+1=Ki,t+α(si,t−ki,t),

{#eq-capacity_update_rule} where ki,t=Ki,t/j​Kj,t is the capacity share of technology i, and α is a small adjustment parameter. If total capacity is normalised or grows proportionally across technologies, we can approximate ki,t≈Ki,t and write

Ki,t+1≈Ki,t+α(si,t−Ki,t),

which moves capacity toward the investment share. In steady state, Ki,t+1=Ki,t implies Ki=si, so that the steady-state capacity shares and investment shares coincide. If the system-cost benchmark satisfies Ki*=si* for some target allocation s*, and the effective costs Ci are calibrated so that (@eq-adjusted_velocity_rule) yields s* when Ki=Ki*, then the pair (K*,s*) constitutes a fixed point of the combined investment–capacity dynamics.

## Summary of the algorithmic adjustment

Under this deliberately simplified framework, a CPAT-style algorithm that uses system cost information can proceed as follows:

- Given marginal system cost functions MSCi(Ki), identify a benchmark capacity mix Ki* (from engineering studies or a separate optimisation).

- At each time step, observe current capacities Ki,t and compute the capacity gaps (Ki,t−Ki*).

- Construct effective costs

Ci,t=Ci,t+qi(Ki,t−Ki*),

- where qi is calibrated from the slope of MSCi around Ki*.

- Apply the adjusted velocity rule

si,t=1N+Ct−Ci,tλ,

- with clipping and renormalisation as needed.

- Update capacities using a simple rule such as

Ki,t+1=Ki,t+α(si,t−Ki,t),

- or another suitable mapping consistent with CPAT’s aggregate capacity dynamics.

Although highly stylised, this scheme shows how system-cost-derived optimal capacities can be integrated into a velocity-style investment rule using only linear-quadratic adjustments that preserve analytical transparency and spreadsheet implementability.

# System-Cost-Driven Investment Rule Without Access to Optimal Capacities

The preceding analysis assumed that the benchmark optimal capacities {Ki*} were known, allowing the construction of effective costs based on capacity gaps. In many practical settings, including CPAT applications, such optimal capacities may not be available. Instead, we may only observe (or estimate) the marginal system cost profiles of technologies as a function of their existing capacities. In this section we derive a simple, fully self-contained adjustment to the velocity rule that operates directly on system cost signals without requiring explicit knowledge of {Ki*}.

## Stylised system cost structure

Suppose system costs for each technology can be represented as follows:

MSCi(Ki)=γC>0,if i=coal,−γG<0,if i=gas,ρiKi2, ρi>0,if i∈{solar, wind, hydro}.

{#eq-stylised_MSCs}

This functional form captures three common system effects:

- Coal imposes a system cost (e.g. inflexibility, pollutant externalities) independent of its level, represented by the constant γC.

- Gas provides a system benefit (flexibility, ramping capability), represented by the negative marginal cost −γG.

- Variable renewables (solar, wind, hydro) impose increasing system costs as their penetration grows, captured by a simple quadratic marginal cost ρiKi2.

Crucially, (@eq-stylised_MSCs) can be evaluated using only current capacity levels, meaning that no optimal solution {Ki*} is required.

## Total system-adjusted marginal cost

To embed these system costs into the investment rule, we define an adjusted marginal cost for technology i:

Ci(Ki)=Ci+η MSCi(Ki),

{#eq-adjusted_cost_no_Kstar} where Ci is the base LCOE and η is a scaling parameter converting system costs (which may be in $/MW or $/MW-year) into units compatible with levelised energy cost.

Thus,

Ccoal=Ccoal+ηγC,

Cgas=Cgas−ηγG,

Cres,i=Cres,i+ηρiKi2.

These adjusted costs reflect the real-time system value/opportunity cost of further investment in each technology.

## Lagrangian formulation and first-order condition

As before, we consider a velocity-style quadratic objective:

min{si}i​Ci(Ki)si+λ2si2 s.t. i​si=1, si≥0.

{#eq-system_cost_velocity_objective}

The Lagrangian is

L(s,μ)=i​Ci(Ki)si+λ2si2−μi​si−1.

The first-order condition for si is unchanged in structure:

Ci(Ki)+λsi−μ=0,

leading to

si=μ−Ci(Ki)λ.

Summing over all i yields the same expression for the multiplier:

μ=C+λN,  C=1Ni​Ci(Ki).

## Updated investment rule

The resulting update rule is:

si=1N+C−Ci(Ki)λ,

{#eq-system_cost_adjusted_velocity_rule} with non-negativity enforced by clipping and renormalisation.

Since Ci(Ki) incorporates the system effects directly, the rule automatically reduces investment in coal due to the penalty γC, increases investment in gas due to the system benefit −γG, and moderates investment in renewables when their existing capacity is high due to the quadratic system burden ρiKi2.

## Capacity update and implicit convergence

If capacities evolve according to the simple update rule

Ki,t+1=Ki,t+α(si,t−Ki,t),

{#eq-no_Kstar_capacity_update} then (@eq-system_cost_adjusted_velocity_rule) creates a self-contained dynamic system: capacities determine system costs, which determine adjusted costs, which determine shares, which update capacities.

A steady state satisfies

Ki=si,

and

Ci(Ki)=C.

Thus the system converges (if stable) to a configuration in which the marginal system-adjusted costs of all active technologies are equalised—even though no explicit optimal capacities {Ki*} were ever required.

## Interpretation

This extension demonstrates that the velocity model can incorporate system cost information even when optimal capacities are unavailable. The system-cost functional forms in (@eq-stylised_MSCs) play an analogous role to the gap term (Ki−Ki*) in the previous section: they modify each technology’s effective cost based on its current penetration in the system. The resulting rule remains analytically transparent, computationally light, and easily implemented in CPAT.

# Mathematical Derivations of Velocity and Acceleration Model Quantities

This appendix provides derivations omitted from the main text for readability.

## Derivation of the velocity model share rule

Consider the problem:

min{si}i​Cisi+λ2si2 s.t. i​si=1.

Form the Lagrangian:

L=i​Cisi+λ2si2−μi​si−1.

Taking first-order conditions yields:

Ci+λsi−μ=0,

so that

si=μ−Ciλ.

Summing over i produces:

i​si=Nμ−i​Ciλ=1,

which implies:

μ=C+λN.

Substituting back yields:

si=1N+C−Ciλ,

as given in the main text.

## Derivation of the acceleration model update rule

Consider the one-period problem:

min{si,t}i​Ci,tsi,t+λ2(si,t−si,t−1)2,

subject to i​si,t=1. Form the Lagrangian:

Lt=i​Ci,tsi,t+λ2(si,t−si,t−1)2−μti​si,t−1.

Differentiating yields:

Ci,t+λ(si,t−si,t−1)−μt=0.

Thus,

si,t=si,t−1+μt−Ci,tλ.

Summing over i yields:

μt=1Ni​Ci,t.

## Alternative Penalty Structures

The penalty framework can be generalised in several directions. For instance, replacing the quadratic penalty with a power penalty:

γ|si|p, p>1,

allows greater control over the curvature of the diversification effect. Alternatively, an entropy penalty:

−ηi​silnsi

yields an allocation similar to the logit model but permits zero shares when η is small. These extensions may be useful where the aim is to mimic optimisation models with explicit diversification constraints.

# Algorithmic Implementation

This appendix describes a practical algorithm for implementing the combined velocity–acceleration allocation rule in a spreadsheet environment such as Excel. The simple models and the combined model lend themselves to simple algorithmic implementation suitable for spreadsheets.

## Background: Simple Velocity model

- Compute the cost average C.

- Evaluate si=1N+C−Ciλ.

- Set negative values to zero.

- Renormalise to ensure i​si=1.

## Background: Simple Acceleration model

- Compute Ct.

- Update shares using si,t=si,t−1+(Ct−Ci,t)/λ.

- Clip negative shares and renormalise.

## Implementation in Excel

The starting point is the closed-form update equation derived in Section 6,

si,t=1N+Ctadj−Ci,tbase−Ci,tsys(st−1;D)+λasi,t−1λv+λa,

{#eq-A3_combined_update_final} where the adjusted average cost is

Ctadj=Ctbase+Ctsys(st−1;D)−λaN.

{#eq-A3_Cbar_adj} Here λv is the velocity penalty, λa is the acceleration penalty (typically set smaller), N is the number of technologies, and D is a country-level population density measure.

The base cost and system adjustments are defined as

Ci,ttot(st,st−1;D)=Ci,tbase+Ci,tsys(st−1;D)+λvsi,t+λa(si,t−si,t−1),

with system costs

Ci,tsys(st−1;D)=−βGGisgas,t−1+κRRiSRES,t−1+ϕiDsi,t−1,

{#eq-A3_Csys} and

SRES,t−1=j​Rjsj,t−1.

In a spreadsheet, the implementation proceeds year by year. The following steps describe the algorithm for period t, given known values of costs, parameters and previous-year shares si,t−1.

Step 1: Set up inputs.

For each technology i and year t, the spreadsheet should contain:

- the base LCOE, Ci,tbase;

- technology indicators Gi (gas), Ri (variable renewables);

- previous-year shares si,t−1;

- common parameters λv, λa, βG, κR, and ϕi;

- population density D.

These can be arranged in a standard technology-by-year matrix, with separate columns for each parameter.

Step 2: Compute aggregate renewable share and system adjustments.

First compute the aggregate share of variable renewables in year t−1:

SRES,t−1=j​Rjsj,t−1.

In Excel this can be implemented as a SUMPRODUCT over the row or column containing Rj and sj,t−1.

Then compute the system adjustment term for each technology using (@eq-A3_Csys):

Ci,tsys(st−1;D)=−βGGisgas,t−1+κRRiSRES,t−1+ϕiDsi,t−1.

This requires (i) reading sgas,t−1 from the previous-year gas share, (ii) multiplying the renewables indicator Ri by SRES,t−1, and (iii) scaling the previous-year share by ϕi/D.

Step 3: Compute average base and system costs.

Compute the simple averages:

Ctbase=1Ni​Ci,tbase,  Ctsys(st−1;D)=1Ni​Ci,tsys(st−1;D),

using an AVERAGE or SUM/N formula across technologies.

Then compute the adjusted average cost as in (@eq-A3_Cbar_adj):

Ctadj=Ctbase+Ctsys(st−1;D)−λaN.

Step 4: Compute preliminary new shares (before clipping).

For each technology, compute the unconstrained share si,traw using (@eq-A3_combined_update_final):

si,traw=1N+Ctadj−Ci,tbase−Ci,tsys(st−1;D)+λasi,t−1λv+λa.

In Excel, this is a single cell formula referencing the relevant row values for Ci,tbase, Ci,tsys, si,t−1 and the scalar parameters λv, λa and N.

Step 5: Apply non-negativity and renormalise.

To enforce si,t≥0, define:

si,t=max{0,si,traw}.

In Excel this is implemented with a MAX formula, for example =MAX(0, s_raw).

Then renormalise the shares so they sum to one:

si,t=si,tj​sj,t.

This requires computing the sum of sj,t across all technologies and dividing each technology’s clipped share by this sum.

Step 6: Iterate over time.

Once the sheet is set up for a single year, the same formulas can be copied across columns to simulate multiple years. For each t+1, the previous-year shares si,t become the inputs si,t in Step 1. The system automatically generates a sequence of investment shares consistent with the combined velocity–acceleration model and the chosen parameters.

Summary.

The combined model can thus be implemented with standard spreadsheet functions (SUM, SUMPRODUCT, AVERAGE, MAX and simple arithmetic), ensuring compatibility with CPAT’s existing Excel-based architecture. The key policy-relevant inputs are the base LCOEs, system-cost parameters (βG,κR,ϕi), the penalty parameters (λv,λa) and the population density D. All derived quantities, including the total effective levelised cost and the evolution of shares, follow mechanically from the above steps.

# Optimal System-Cost-Based Investment and Retirement Rules

This appendix restates the yearly model in a simple, stylised form and derives the optimal retirement and investment rules under the assumption that all decisions are driven by system costs rather than by an explicitly computed optimal capacity mix. System costs are taken as given functions of capacity, and the allocation of new investment responds to effective costs using a combined velocity–acceleration rule. Retirement responds to the same effective costs through a simple static optimisation. The framework is annual (one step per year) and does not rely on 8,760-hour operational detail.

## System costs and effective annual costs

Let i index technologies and t index years. Let Ki,t denote installed capacity of technology i at the start of year t, and let Ci denote its base levelised cost of electricity (LCOE), assumed constant over time for simplicity.

We assume stylised system costs per unit of capacity given by

MSCi(Ki,t)=γC>0,if i=coal,−γG<0,if i=gas,ρiKi,t2, ρi>0,if i∈{solar, wind, hydro},

{#eq-D_MSCs_opt} where γC represents a constant system penalty for coal, −γG a constant system benefit for gas (flexibility), and ρiKi,t2 an increasing system burden from variable renewables as their penetration grows.

We convert these system costs into an effective annual cost per MWh:

Ci,teff=Ci+η MSCi(Ki,t),

{#eq-D_Ceff_opt} where η is a scaling parameter aligning units. The effective cost Ci,teff is the key object that drives both retirement and investment.

We also define the average effective cost across all N technologies:

Cteff=1Nj=1NCj,teff.

{#eq-D_Cbar_opt}

For generation, let CFi denote the (constant) capacity factor of technology i. Total generation in year t is

Gt=i​CFiKi,t,

and we treat required generation Gtreq as exogenous. The system must choose retirements and investments so that generation after retirement and investment satisfies

i​CFiKi,t+1=Gtreq.

## Simplified System-Cost-Driven Retirement and Investment

This appendix provides a concise, stylised yearly model in which investment and retirement respond to system-adjusted effective costs. All quantities are annual; no hourly structure is used.

Let Ki,t denote installed capacity of technology i at the start of year t, and Ci its base LCOE. System costs are given exogenously as:

MSCi(Ki,t)=γC>0,if i=coal,ρiKi,t2,if i∈R (solar, wind, hydro).

Effective marginal cost (per MWh) is

Ci,teff=Ci+η MSCi(Ki,t).

Let CtRES,min=minj∈RCj,teff denote the cheapest renewable effective cost at time t. Coal has a (given) variable marginal cost CC,tvar.

## Optimal investment rule with velocity and acceleration

After retirement, capacity is Ki,tpre-inv. Let the required generation in year t+1 be Gt+1req. The total capacity addition needed to meet the future requirement is determined externally (for example, from demand projections and capacity factors). Denote the total new capacity by It and its allocation across technologies by Ii,t, so that

i​CFiIi,t=Gt+1req−i​CFiKi,tpre-inv.

Define si,t=Ii,t/It as the share of new capacity investment going to technology i, so that i​si,t=1.

We now derive the optimal investment shares by solving a quadratic problem that combines effective costs with velocity and acceleration penalties. The planner chooses {si,t} to minimise:

min{si,t}i​Ci,teffsi,t+λv2si,t2+λa2(si,t−si,t−1)2 s.t. i​si,t=1,

{#eq-D_inv_opt_problem} where λv>0 is the velocity penalty parameter and λa≥0 is the acceleration penalty parameter. This problem is fully analogous to the combined velocity–acceleration formulation presented earlier, but now the costs entering the objective are the effective costs Ci,teff derived from system costs.

Form the Lagrangian:

Linv=i​Ci,teffsi,t+λv2si,t2+λa2(si,t−si,t−1)2−μti​si,t−1,

where μt enforces the share constraint. The first-order condition with respect to si,t is

Ci,teff+λvsi,t+λa(si,t−si,t−1)−μt=0.

{#eq-D_inv_FOC} Collecting terms in si,t gives:

(λv+λa)si,t=μt−Ci,teff+λasi,t−1.

{#eq-D_inv_linear} Thus,

si,t=1λv+λaμt−Ci,teff+λasi,t−1.

{#eq-D_inv_pre_mu}

To determine μt, sum (@eq-D_inv_pre_mu) over i, using i​si,t=1 and i​si,t−1=1:

1=Nμt−i​Ci,teff+λai​si,t−1λv+λa=Nμt−i​Ci,teff+λaλv+λa.

Therefore,

μt=Cteff−λaN+λv+λaN,

where Cteff is the average effective cost defined in (@eq-D_Cbar_opt).

Substituting back into (@eq-D_inv_pre_mu) yields the closed-form optimal investment rule:

si,t=1N+Cteff−Ci,teff+λa(si,t−1−1N)λv+λa.

{#eq-D_inv_rule_final}

When λa=0, this collapses to a pure velocity rule:

si,t=1N+Cteff−Ci,teffλv,

and when λv is large relative to λa, the rule closely tracks si,t−1 with only gradual adjustments driven by cost differentials.

In practice, negative si,t values are clipped to zero and the remaining positive shares are renormalised to sum to one.

## Capacity update with optimal retirement and investment

After choosing optimal retirements and investments, capacity evolves according to

Ki,t+1=Ki,tpre-inv+Ii,t=(Ki,t−Ri,t)+Itsi,t,

{#eq-D_K_update_opt} where Ri,t solves the retirement problem (i.e. merit-order retirement based on Ci,teff), and si,t solves the investment problem (@eq-D_inv_opt_problem) (velocity–acceleration optimum based on Ci,teff).

The total investment It is chosen (outside this appendix) to ensure that the generation constraint

i​CFiKi,t+1=Gt+1req

is satisfied.

## Simplified algorithmic implementation

The optimal yearly algorithm, in stylised form, proceeds as follows.

Given at the start of year t: capacities Ki,t, previous investment shares si,t−1, base LCOEs Ci, capacity factors CFi, system-cost parameters γC,γG,ρi, scaling η, velocity and acceleration parameters λv,λa, and generation requirements Gtreq,Gt+1req.

Step 1: Compute effective costs.

- Compute system costs:

MSCi(Ki,t) from (@eq−DMSCsopt),

Ci,teff=Ci+η MSCi(Ki,t),

- Compute their average:

Cteff=1Nj​Cj,teff.

Step 2: Retirement (coal-only, cost-triggered with velocity smoothing).

- Compute the cheapest renewable effective cost:

CtRES,min=minj∈RCj,teff.

- Check whether coal is economically dominated:

CC,tvar>CtRES,min.

- If this condition is false, coal retires only at its technical rate:

rC,t=δC.

- If the condition is true, compute the velocity-based additional retirement incentive:

uC,tvel=min​uCmax,CC,tvar−CtRES,minΛR,  uC,tvel≥0.

- Smooth the retirement change using an acceleration-like rule:

uC,t=(1−αR)uC,t−1+αRuC,tvel.

- The total coal retirement fraction is:

rC,t=δC+uC,t,  rC,t∈[0,1].

- For all non-coal technologies i≠C, retirement is purely technical:

ri,t=δi.

- Apply retirement to obtain pre-investment capacities:

Ki,tpre-inv=(1−ri,t)Ki,t.

Step 3: Required net investment.

- Compute the generation from the pre-investment fleet:

Gtpre-inv=i​CFiKi,tpre-inv.

- Compute the required additional generation:

ΔGt=Gt+1req−Gtpre-inv.

- Translate ΔGt into total new capacity It (e.g. using an average capacity factor across new investments or a separate simple rule).

Step 4: Optimal investment shares (velocity–acceleration rule).

- Compute the closed-form optimal shares:

si,t=1N+Cteff−Ci,teff+λa(si,t−1−1N)λv+λa,

- Clip negative si,t to zero and renormalise so that i​si,t=1.

Step 5: Capacity update.

- Allocate new capacity:

Ii,t=Itsi,t.

- Update capacities:

Ki,t+1=(Ki,t−Ri,t)+Itsi,t.

Step 6: Repeat.

- Proceed to year t+1 with updated capacities Ki,t+1 and shares si,t.

This completes the stylised optimal model. All decisions—retirement and investment—are driven by effective costs that combine base LCOE and exogenous system cost functions. Planned retirement is determined by exogenous exponential decay for non-coal and planned retirement for coal. Early retirement exists only for coal if the variable cost is more than the total cost of renewables. Investment shares are determined by a velocity–acceleration optimisation, both of which are analytically transparent and compatible with a yearly, spreadsheet-based CPAT implementation.

# References